Tender description – TES/FSV430/101-EN

Vortex flowmeter

VortexMaster FSV430

Pos. 1: Vortex flowmeter – VortexMaster FSV430.

- Common and user friendly handling accross the measurement portfollio (e.g. flow, pressure, temperature, level, etc.)

- Accuracy for liquid: 0.65 % of rate (volume flow)

- Accuracy for gas & steam: 0.9 % of rate (volume flow)

- Installation length according to NAMUR for direct replacement of electromagnetic flowmeter

- Integrated self verification according to NAMUR NE107

  - Sensor and transmitter self check routines

  - Continuous readback loop check for current output

- Option for temperature measurement/compensation available

- Simple spare part management with minimum of components/sensors: maximum 1 sensor for all pipe sizes and 1 transmitter

- Display operation through the glass without opening the cover

- Easy replacement of transmitter components without reprogramming e.g. with SensorMemory

- Simple and fast commisionning:

  - Easy setup for the most important parameters

  - Clear text setup directly on the transmitter

  - Simulation modes for all input and outputs

- Technical data – Sensor 

Medium:		…………………………………………………………….....
Measuring range [incl. Unit]: 	………………………………................................................

| - Sensor design | - Flange DIN | - Wafer DIN (65mm lay length) |
| --- | --- | --- |
|  | - Flange ASME | - Wafer ASME |

- Meter size: 	DN/ANSI …………………………………………………...
Pressure rating:	PN/CL ………………………….…..................................

- Wetted parts: 	Stainless steel, or others ................................................

- Protection class:	IP 66/67/NEMA 4X
Sensor design: 	Vibration compensated sensor with long term stability

- Temperature measurement:	Required: YES / NO

Technical data – Transmitter

Operation:	Intuitive via glass cover (TTG), capactive keys immune against

polution

Display: 		Multiline display, possibility to simultaneously display of e.g. flow, temperature and totalizer

Commissioning: 	EasySetup with guided user interface via display or laptop

| - Output/Communication: | - 4 ... 20 mA, HART protocol [HART 7] |
| --- | --- |
|  | - Modbus RTU-RS485 |
|  | - Profibus PA |
|  | - FOUNDATION Fieldbus |

| - Digital output configurable as: | - None |
| --- | --- |
|  | - Pulse |
|  | - Frequency |
|  | - Logic |

| Hazardous approvals: | Approval | Protection category |
| --- | --- | --- |
|  | - ATEX | - Ex nA/tc [NI/DIP] – Zone 2/22 |
|  | - FM | - Ex ia/ta [IS/DIP] – Zone 0/20 // 1/21 |
|  | - IECEX | - Ex d/tb [XP/DIP] – Zone 1/21 |

Measuring medium temperature:	-55 … 280°C [-67 … 536°F]; optional: -55 …400°C [-67 … 750°F]

| - Ambient temperature: | - -20 … +85°C [-4 … 185 °F] |
| --- | --- |
|  | - -40 … +85°C [-40 … 185 °F] |

Protection class:	IP 66/67/NEMA 4X

Diagnostic:	Acc. Namur NE107, Status messages including helptext available on display and by digital communication

Verification capability: 	ABB Ability SRV500 or equivalent

Device name: 	VortexMaster FSV430 or equivalent

Producer: 	ABB Automation Products GmbH

Additional technical information:

Internet

Homepage VortexMaster FSV400